# ppt 1

## Slide 1 — Title

### On slide

FROM THE PHYSICAL WORLD
 TO YOUR FIRST IoT SYSTEM

Subtitle:

Applied IoT | Module 1 — Foundations

Small visual:

Physical World → Electronics → Intelligence → Action

### Visual

Hero image/collage:

- Robot

- Smart agriculture

- Smart home

- Industrial automation

Keep text extremely minimal.

### Instructor intent

Set the expectation:

"Until now, your programs lived on a screen. Today we're going to make that software interact with the physical world."

# Slide 2 — The Hook

### Title

How does a street light know when to turn ON?

### On slide

Large image of a street light.

Three questions appearing sequentially:

1. How does it know it's dark?

2. Who makes the decision?

3. How does the lamp actually turn ON?

### Visual

Streetlight → sensor → controller → lamp.

Don't reveal the labels initially. Let students answer.

### Instructor interaction

Ask the class before revealing the answer.

# Slide 3 — Software vs Physical World

### Title

When Software Touches the Real World

### On slide

Two halves:

Software

Keyboard → Program → Screen

versus

Physical Computing

World → Sensor → Program → Actuator → World

Bottom:

Inputs become physical. Outputs become physical.

### Visual

Split-screen illustration.

This comes directly from the RM's distinction between software on a screen and software interacting with physical inputs/outputs.

# Slide 4 — What is Physical Computing?

### Title

Physical Computing

### On slide

Large diagram:

PHYSICAL WORLD

↓

SENSE

↓

COMPUTE

↓

ACT

↓

PHYSICAL WORLD

Small caption:

Hardware connects software to the physical world.

### Visual

Circular loop rather than a straight flow.

### Key message

Don't over-explain. Instructor explains.

# Slide 5 — The Core Mental Model

### Title

The Loop Behind Physical Systems

### On slide

PHYSICAL WORLD

↓     ↑

SENSE  ACTUATE

↓     ↑

COMPUTE

Center:

### Sense → Compute → Actuate

### Visual

Make this the hero diagram of PPT-1.

Use icons:

👁️ Sense
 🧠 Compute
 ⚙️ Actuate

The RM explicitly identifies this as the central mental model.

# Slide 6 — What Happens at Each Stage?

### Title

What Does Each Stage Actually Do?

Three large cards:

### SENSE

Physical quantity
 ↓
 Electrical signal

### COMPUTE

Signal
 ↓
 Decision

### ACTUATE

Command + Power
 ↓
 Physical effect

Bottom:

Light → Sensor → Decision → Lamp

### Visual

Three-stage animated/reveal diagram.

The RM gives exactly this conceptual breakdown.

# Slide 7 — Let's Decode a Street Light

### Title

Let's Trace the Loop

### On slide

Streetlight system:

Ambient Light

↓

Light Sensor

↓

Voltage

↓

Controller

↓

Decision

↓

Lamp

### Highlight

SENSE → COMPUTE → ACTUATE

### Interaction

Ask:

Which part is sensing?

Which part is computing?

Which part is acting?

# Slide 8 — Find the Loop

### Title

Can You Find the Loop?

Show 3–4 real-world images:

- Automatic irrigation

- Smart AC

- Warehouse robot

- Wearable

Under each:

SENSE | COMPUTE | ACTUATE

But leave them blank.

### Activity

Students identify the three.

### Visual

Four image cards.

The RM specifically provides these examples.

# Slide 9 — From Physical System to IoT

### Title

What Makes It "IoT"?

Start with:

SENSE → COMPUTE → ACTUATE

Then introduce:

SENSE → COMPUTE → ACTUATE

↕

NETWORK

### Big message

IoT adds communication.

### Visual

Device → Network → Cloud/Other Device.

The RM explicitly frames IoT as adding communication to the physical loop.

# Slide 10 — Embedded System vs IoT

### Title

Same Device. More Capability.

Two diagrams:

### Standalone

Sensor → Controller → Actuator

### Connected

Sensor → Controller → Actuator

↕

Network

### Question

What can the connected system do that the standalone system cannot?

Reveal:

- Remote monitoring

- Remote control

- Data collection

- Coordination

# Slide 11 — AIoT & Physical AI

### Title

What Happens When We Add Intelligence?

Visual stack:

Physical Computing

↓

IoT

↓

AIoT

↓

Physical AI

Minimal definitions:

IoT
 Connect things.

AIoT
 Add AI to connected things.

Physical AI
 AI that perceives, reasons and acts in the physical world.

### Visual

Robot / autonomous vehicle / smart factory.

The RM uses this layered model while noting that the categories can overlap.

# Slide 12 — Your First Challenge

### Title

Analyze This System

Show a large image/illustration of an automatic irrigation system.

Ask:

- What does it sense?

- What does it compute?

- What does it actuate?

- What happens if the sensor fails?

### Interaction

Give students ~2 minutes.

This becomes the PPT version of the RM's real-system analysis activity.

# Slide 13 — The Hidden Foundation

### Title

But What Carries All of This?

Show:

Physical World

↓

Sensor

↓

─ ELECTRICITY ─

↓

Microcontroller

↓

─ ELECTRICITY ─

↓

Actuator

↓

Physical World

Big statement:

Every arrow eventually depends on electrical behaviour.

This is the transition from IoT concepts into circuits. The RM makes this exact connection.

# Slide 14 — What Is a Circuit?

### Title

Before We Build IoT Systems, We Need a Circuit

Visual progression:

Battery → Wire → LED → Wire → Battery

Then:

Closed path → Current can flow

Show:

### Open circuit ❌

broken path

### Closed circuit ✅

complete path

### Quick question

Which one will light the LED?

# Slide 15 — Three Ideas You Must Understand

### Title

Voltage. Current. Resistance.

Three large visual cards:

### VOLTAGE

The push

V

### CURRENT

The flow

I

### RESISTANCE

The opposition

R

Bottom:

These three explain a huge part of circuit behaviour.

The RM deliberately introduces these concepts through intuition before formulas.

# Slide 16 — Water Analogy

### Title

A Useful Intuition

Visual:

| Water | Electricity |
| --- | --- |
| Pressure | Voltage |
| Flow rate | Current |
| Narrow pipe | Resistance |

But don't put a huge table.

Instead visually show:

Pressure → Pipe → Flow

beside:

Voltage → Resistance → Current

### Small note

Analogy ≠ exact physics

The RM explicitly labels the water comparison as a teaching model.

# Slide 17 — Meet Ohm's Law

### Title

How Are V, I and R Related?

Start with:

Voltage

↓

Resistance

↓

Current

Then reveal:

# V = I × R

Underneath:

I = V / R

### Visual

Make the equation the dominant object on the slide.

# Slide 18 — Predict Before You Calculate

### Title

What Happens to Current?

Show:

5 V ─── [ 1 kΩ ] ───

I = 5 mA

Then:

5 V ─── [ 2 kΩ ] ───

Question:

### What happens to current?

Options:

A. Doubles
 B. Stays same
 C. Halves

### Interaction

Students vote/predict first.

Then reveal:

5 mA → 2.5 mA

This directly reinforces the RM's "predict before you build" approach.

# Slide 19 — The LED Problem

### Title

Why Can't We Connect an LED Directly?

Visual:

### ❌

3.3 V → LED → GND

versus

### ✅

3.3 V → RESISTOR → LED → GND

Question:

What is the resistor doing?

Reveal:

### It limits current.

The RM explains that an LED does not behave like an ordinary resistor and needs a current-limiting resistor.

# Slide 20 — Let's Simulate It

### Title

🧪 Let's Build It in Wokwi

### On slide

Goal

Build a simple LED circuit and observe the effect of resistance.

### Components

- DC supply

- LED

- Resistor

- Wires

### Visual

Large screenshot/diagram of the intended circuit.

### Instructor

Open Wokwi and demonstrate.

This is where simulation becomes part of the story rather than a separate project.

# Slide 21 — Predict → Simulate → Observe

### Title

What Do You Predict?

Show the same circuit.

Then:

### Experiment 1

1 kΩ

### Experiment 2

2 kΩ

### Ask:

What changes?

- Current?

- Brightness?

Then:

Run the simulation.

### Visual

Two circuit states side by side.

# Slide 22 — Series Circuits

### Title

One Path → Series

Visual:

┌── R1 ── R2 ──┐

│              │

└──── 5 V ─────┘

Three simple rules:

One path

Same current

Voltage is shared

Then:

Rtotal = R1 + R2

### Visual

Use arrows showing the single current path.

# Slide 23 — Parallel Circuits

### Title

Multiple Paths → Parallel

Visual:

┌── R1 ──┐

5 V ──────┤        ├────

└── R2 ──┘

Three rules:

Multiple paths

Same voltage

Current splits

### Quick question

What happens if one branch opens?

Reveal:

Other branches can continue working.

# Slide 24 — Series vs Parallel

### Title

Can You Predict the Difference?

Use a visual comparison rather than a dense table.

| Series | Parallel |
| --- | --- |
| One path | Multiple paths |
| Same current | Same voltage |
| Voltage divides | Current divides |
| One break → all stop | One branch can fail independently |

### Challenge

Show two circuits with LEDs and ask:

Which circuit keeps the other LED ON if one LED fails?

# Slide 25 — KVL & KCL

### Title

Two Laws That Let You Check Your Circuit

Split screen.

### KVL

Around a loop

Voltage IN = Voltage OUT

Visual:

+5V → -2V → -3V → 0

### KCL

At a junction

Current IN = Current OUT

Visual:

13 mA

→

20 mA →  ●

→

7 mA

Bottom:

They help you check your reasoning.

The RM explicitly presents KVL and KCL as independent checks on circuit reasoning.

# Slide 26 — Power

### Title

Voltage + Current = Power

Large:

# P = V × I

Visual:

Electrical Energy → Component → Heat / Light / Motion

Then example:

5 V × 20 mA = 100 mW

### Question

What happens to power if current increases?

Keep the explanation short; the RM carries the detailed treatment.

# Slide 27 — Circuit Safety

### Title

Before You Power It...

Use 5 large icons:

🔌 Power off before wiring

⚡ Check voltage

🔗 Check connections

💡 Use a resistor with LEDs

🔥 Stop if something gets hot

Bottom:

### Predict before you power.

The RM's safety section specifically emphasizes powering off, checking polarity/ratings, avoiding shorts, and predicting before powering.

# Slide 28 — The Engineer's Habit

### Title

Don't Guess. Debug.

Large flow:

# Observe

↓

# Identify

↓

# Check

↓

# Test

↓

# Change

↓

# Retest

Then:

What should happen?
 What actually happened?
 What could explain the difference?

This directly follows the RM's troubleshooting methodology.

# Slide 29 — Circuit Prediction Challenge

### Title

🧠 Predict Before You Run

Show a circuit with:

- 5 V

- R1 = 1 kΩ

- R2 = 4 kΩ

- Series connection

Ask:

### 1. What is the total resistance?

### 2. What is the current?

### 3. What voltage appears across each resistor?

Give students ~2 minutes.

Then progressively reveal:

Rtotal = 5 kΩ

I = 1 mA

V1 = 1 V

V2 = 4 V

This is the PPT realization of the Circuit Prediction Challenge from the Master Sheet.

# Slide 30 — The Big Picture

### Title

Everything Connects

Visual:

PHYSICAL WORLD

↕

SENSE

↓

COMPUTE

↓

ACTUATE

↕

PHYSICAL WORLD

↕ COMMUNICATE

↕ AI

↕ AUTOMATION

And underneath:

Electrical signals

+

Electrical power

↓

Physical systems

### Message

IoT is not just software.
 It is software + electronics + the physical world.

# Slide 31 — What Can You Do Now?

### Title

You Can Now...

Use capability cards:

### 🌍 Analyze

Break a physical system into
 Sense → Compute → Actuate

### ⚡ Reason

Understand
 Voltage → Current → Resistance

### 📐 Predict

Use
 Ohm's Law + Series + Parallel + KVL + KCL

### 🧪 Test

Build and experiment in Wokwi

### 🐛 Debug

Observe → Check → Test → Retest

### 🔒 Work Safely

Predict before powering.

These capabilities directly reflect the RM learning outcomes.

# Slide 32 — The Road Ahead

### Title

From Circuits to IoT

Visual progression:

Circuit

↓

ESP32

↓

Sensors

↓

Actuators

↓

Communication

↓

Cloud

↓

AI

↓

AIoT System

Highlight only:

### Today

Circuit Thinking

### Next

Building with the ESP32

Don't explain future topics deeply.

This gives students a reason to continue without prematurely teaching PPT-2/PPT-3.

# ppt2

# PPT-2 Flow Plan

## From Components to a Microcontroller-Controlled System

Target: ~60 minutes
 Target slides: 28–32

## Section 1 — Connect to PPT-1

### Slide 1 — Title

FROM COMPONENTS TO A MICROCONTROLLER-CONTROLLED SYSTEM

Subtitle:
 Understanding the building blocks of IoT

Visual:
 A complete physical system:

Sensor → Microcontroller → Actuator

with components surrounding it.

### Slide 2 — Where Are We Now?

Title:
 You Know the Circuit. Now Open It.

Visual:

PPT-1

Circuit behaviour

Voltage → Current → Resistance

↓

PPT-2

What's actually inside the circuit?

Show a PCB/circuit image with components highlighted.

Message:

A circuit is a team of components. Each component has a job.

### Slide 3 — The Three Boxes

Title:
 Three Boxes. Three Jobs.

Large:

SENSOR

↓

MICROCONTROLLER

↓

ACTUATOR

Underneath:

Bring information in → Make decisions → Create an effect

This becomes the central visual model for PPT-2.

# Section 2 — Electronic Components

### Slide 4 — A Circuit Is a Team

Show a PCB/circuit board with 6 components highlighted:

- Resistor

- LED

- Diode

- Capacitor

- Button

- Transistor

Ask:

Can you identify what each one does?

Don't reveal the answers immediately.

### Slide 5 — Component Jobs

Title:
 Every Component Has a Job

Visual six cards:

| Component | Job |
| --- | --- |
| Resistor | Limit / control |
| Diode | One-way current |
| Capacitor | Store / smooth |
| Button | Open / close |
| LED | Produce light |
| Transistor | Control larger current |

Keep the explanations extremely short.

The detailed explanations remain in the RM.

### Slide 6 — Resistor

Title:
 Resistor — Control the Current

Visual:

Voltage → [ RESISTOR ] → Current

Then three visual applications:

Limit current

Divide voltage

Set default input level

### Quick question

Why did we need a resistor with our LED in PPT-1?

This intentionally connects back to the previous session.

### Slide 7 — Read the Resistor

Title:
 Can You Read This Resistor?

Large physical resistor image.

Show:

Brown | Black | Red | Gold

Ask:

What value is this?

Reveal:

### 1 kΩ ±5%

Then briefly show:

10 × 100 = 1000 Ω

The RM includes resistor colour-code reading and the worked 1 kΩ example.

### Slide 8 — Diode

Title:
 Diode — One-Way Traffic

Visual:

→ |

Use a water-valve analogy visually.

Then:

Forward → Conducts

Reverse → Blocks

Show a diode symbol and physical diode.

### Slide 9 — LED = Light-Emitting Diode

Title:
 An LED Is a Diode That Makes Light

Visual:

Current → LED → Light

Highlight:

- Polarity

- Forward voltage

- Current limitation

Question:

What happens if we remove the resistor?

Connect directly back to PPT-1.

The RM emphasizes that LED brightness is driven by current and that a series resistor is required.

### Slide 10 — Capacitor

Title:
 Capacitor — A Tiny Energy Reservoir

Visual analogy:

Supply ↔ [ CAPACITOR ] ↔ Load

Show charging/discharging curve.

Three applications:

- Smooth power

- Filter signals

- Timing

Don't dive deeply into the RC equation yet.

### Slide 11 — Button

Title:
 A Button Is an Input

Visual:

NOT PRESSED → OPEN

PRESSED     → CLOSED

Then:

Button

↓

Electrical path

↓

Microcontroller

Question:

Does the button generate a voltage?

Answer:

No. It changes the electrical path.

### Slide 12 — The Floating Input Problem

Title:
 What Happens When Nothing Is Connected?

Visual:

3.3 V

|

Button

|

GPIO ───── ????

Show:

### Floating input

HIGH? LOW? HIGH? LOW?

Then introduce:

Pull-up / Pull-down

Keep the detailed electrical reasoning in the RM.

### Slide 13 — Transistor

Title:
 What If the Load Needs More Current?

Visual:

Microcontroller

│

small signal

↓

[TRANSISTOR]

│

↓

MOTOR / PUMP

Big message:

Small signal → Large current

This is one of the most important concepts in the entire PPT.

The RM explicitly frames the transistor as a small-signal-controlled switch for larger loads.

### Slide 14 — The Low-Side Switch

Title:
 How a Microcontroller Controls a Motor

Visual:

+ Supply

│

LOAD

│

TRANSISTOR

│

GND

GPIO ──► Control

Add:

Common Ground

Control resistor

Correct transistor

Do not overload the slide with transistor theory.

### Slide 15 — What Protects the Transistor?

Title:
 Why Is There a Diode Across the Motor?

Show motor coil switching:

ON  → Current flows

OFF → ⚡ Voltage spike

Then:

┌── Diode ──┐

│           │

MOTOR      TRANSISTOR

Big message:

### Flyback diode = protection

This is a great visual "aha" moment.

# Section 3 — Sensors & Actuators

### Slide 16 — Inputs vs Outputs

Title:
 Which Direction Is the Information Moving?

Visual:

PHYSICAL WORLD

↓

SENSOR

↓

INPUT

↓

MICROCONTROLLER

↓

OUTPUT

↓

ACTUATOR

↓

PHYSICAL WORLD

Emphasize:

Input/output is from the microcontroller's point of view.

This distinction is explicitly made in the RM.

### Slide 17 — Sensors

Title:
 Sensors Turn the World Into Signals

Visual:

Light       ─┐

Temperature ─┤

Motion      ─┤ → SENSOR → Electrical signal

Distance    ─┤

Moisture    ─┘

Message:

A sensor converts a physical quantity into something the system can process.

### Slide 18 — Three Ways a Sensor Talks

Title:
 Not All Sensor Outputs Look the Same

Three visual columns:

### Digital

HIGH / LOW

### Analog

0 → continuous range

### Digital Data

Sensor → numerical message

Examples below each.

The RM distinguishes these three output types.

Important: Don't teach ADC/I2C/SPI in depth here. Just establish the categories.

### Slide 19 — What Should You Ask About a Sensor?

Title:
 Choosing a Sensor Is an Engineering Decision

Show five questions around a sensor:

What does it measure?

What range?

How accurate?

What signal?

What voltage/current does it need?

This converts the RM's sensor-selection section into a practical engineering mindset.

### Slide 20 — Actuators

Title:
 Actuators Turn Decisions Into Effects

Visual:

Electrical signal + Power

↓

ACTUATOR

↓

Light / Sound / Motion / Heat

Show examples:

- LED

- Buzzer

- Motor

- Servo

- Pump

- Relay

### Slide 21 — Sensor or Actuator?

Title:
 Which Side Does It Belong To?

Show six images:

- LDR

- Button

- Temperature sensor

- LED

- Buzzer

- Motor

Students classify:

### INPUT or OUTPUT?

Then reveal.

This becomes the Sensor/Actuator Matching activity from the curriculum.

# Section 4 — The Microcontroller

### Slide 22 — The Decision Maker

Title:
 The Microcontroller

Visual:

INPUTS

↓

┌────────────────────┐

│  MICROCONTROLLER   │

│                    │

│  Process           │

│  Memory            │

│  Peripherals       │

└────────────────────┘

↓

OUTPUTS

Message:

The microcontroller connects software to electronics.

### Slide 23 — Microcontroller vs Laptop

Title:
 Why Not Just Use a Laptop?

Visual comparison:

Laptop

🖥️ General-purpose
 📱 Many applications
 💾 Large memory
 ⚡ Higher power

vs

Microcontroller

🔧 Dedicated control
 📌 Direct GPIO
 🔋 Low power
 📦 Small

The RM provides this comparison explicitly.

### Slide 24 — What's Inside?

Title:
 Open the Microcontroller

Visual:

┌──────────────────────────────┐

│       MICROCONTROLLER        │

│                              │

│  CPU    MEMORY    PERIPHERALS│

│   🧠       💾          🔌     │

│                              │

└──────────────────────────────┘

Then one-line roles:

CPU → runs instructions

Memory → stores program/data

Peripherals → connect to the outside world

### Slide 25 — The ESP32-S3

Title:
 Our Microcontroller: ESP32-S3

Visual:

Large ESP32-S3 development board.

Call out:

- Processor

- Memory

- GPIO

- Wi-Fi

- Bluetooth

- Peripherals

Do not turn this into a specification sheet.

The RM uses the ESP32-S3 as the course example and notes its GPIO/peripheral capabilities.

### Slide 26 — GPIO: Where Code Meets Hardware

Title:
 GPIO = General-Purpose Input/Output

Visual split:

### OUTPUT

Code

↓

GPIO HIGH

↓

LED ON

### INPUT

Button

↓

GPIO

↓

Code

This is the most important microcontroller concept of PPT-2.

The RM describes GPIO as the point where software becomes an electrical signal and vice versa.

### Slide 27 — One Pin, Two Roles

Title:
 A GPIO Can Listen or Act

Visual:

GPIO

/      \

INPUT      OUTPUT

↓          ↓

BUTTON       LED

Ask:

If the button is pressed, what does the microcontroller receive?

If the GPIO goes HIGH, what happens to the LED?

This prepares students for PPT-3 without teaching CircuitPython deeply.

# Section 5 — Integration

### Slide 28 — From Button to LED

Title:
 Now Put Everything Together

Visual:

BUTTON

↓

GPIO INPUT

↓

MICROCONTROLLER

↓

GPIO OUTPUT

↓

RESISTOR

↓

LED

Then:

### Input → Decision → Output

This is the first genuinely complete microcontroller-controlled system.

### Slide 29 — From Code to Physical Behaviour

Title:
 How Does One Line of Code Become Light?

Progressively reveal:

button.value

↓

Program decision

↓

GPIO = HIGH

↓

3.3 V signal

↓

Current flows

↓

LED lights

This is an excellent bridge between software and hardware.

The RM provides this exact conceptual connection with its CircuitPython example.

### Slide 30 — The Complete Signal Path

Title:
 Trace the Signal

Use an automatic night-light diagram:

LIGHT

↓

LDR

↓

VOLTAGE

↓

MICROCONTROLLER INPUT

↓

PROGRAM

↓

DECISION

↓

GPIO OUTPUT

↓

LED

↓

LIGHT

Ask:

Where does the physical world become data?

Where does data become physical action?

This is probably the key conceptual slide of PPT-2.

The RM traces exactly this night-light signal path.

### Slide 31 — Design Challenge

Title:
 Choose the Right Parts

Scenario:

A door should detect when it is open and alert someone.

Give students:

Inputs

- Camera

- Magnetic door switch

- Ultrasonic sensor

Outputs

- LED

- Buzzer

- Motor

Ask:

### Which combination would you choose? Why?

Then discuss:

Magnetic switch + LED + buzzer

The RM specifically uses a door-left-open example to demonstrate choosing the simplest suitable parts.

### Slide 32 — Debugging a Physical System

Title:
 The LED Isn't Turning ON. Where Is the Problem?

Visual pipeline:

Program

↓

GPIO

↓

Wiring

↓

Component

↓

Power / Ground

Ask students to identify where they would check first.

Then reveal:

Program running?

↓

GPIO changing?

↓

Wiring correct?

↓

Component okay?

↓

Power/GND okay?

The RM explicitly gives this troubleshooting order.

# Slide 33 — The Complete Mental Model

Title:
 Open the Boxes

Bring everything together:

PHYSICAL WORLD

↓

SENSOR

↓

INPUT

↓

┌─────────────────┐

│ MICROCONTROLLER │

│                 │

│ CPU             │

│ MEMORY          │

│ PERIPHERALS     │

│ GPIO            │

└─────────────────┘

↓

OUTPUT

↓

ACTUATOR

↓

PHYSICAL WORLD

Around the circuit, show:

Resistor | Diode | Capacitor | Button | Transistor

This is the hero recap visual.

# Slide 34 — What Can You Do Now?

Title:
 You Can Now...

Five capability cards:

### 🔧 Identify

Recognize common components.

### 🔍 Trace

Follow a signal through a system.

### 🌡️ Classify

Sensor vs actuator.

### 🧠 Understand

What the microcontroller does.

### ⚡ Connect

GPIO ↔ physical hardware.

# Slide 35 — The Road Ahead

Title:
 Now We Make It Run

Visual:

Components

↓

Microcontroller

↓

GPIO

↓

CircuitPython

↓

ESP32-S3

↓

Your First Program

Big statement:

Next: Your first ESP32 program.

This is where PPT-3 begins, so don't teach CircuitPython here.

# PPT-2 overall flow

| Slides | Section | Purpose |
| --- | --- | --- |
| 1–3 | Connection + mental model | Connect to PPT-1 |
| 4–15 | Components | Understand what each component does |
| 16–21 | Sensors & Actuators | Understand physical inputs/outputs |
| 22–27 | Microcontroller | Understand the "Compute" box |
| 28–30 | Integration | Connect input → compute → output |
| 31 | Design challenge | Choose components for an application |
| 32 | Debugging | Apply engineering thinking |
| 33–35 | Recap + next step | Consolidate capability |

### One important pedagogical decision

For PPT-2, I would not introduce a big Wokwi project yet.

The RM's content is primarily about component recognition, system architecture, and understanding the microcontroller. The curriculum's hands-on target is Build basic circuits + Sensor/actuator matching + Microcontroller exploration.

So the practical moments should be small and embedded:

Identify → Match → Trace → Predict → Build a simple circuit → Explain

Then PPT-3 can make the ESP32 + CircuitPython the main hands-on experience.

That keeps the progression clean:

PPT-1: Why does a physical system work?
 PPT-2: What are the parts that make it work?
 PPT-3: How do I program the system?

# ppt3